ANNEX 7e

Agenda Item 11.6 aj) (& ag))

F3P Schedule of Manoeuvres

SCHEDULE F3P-AP		K-Factor

AP 01:	Lying square eight with ½ rolls in all vertical (up and down) lines  (Centre manoeuvre) 	4

AP 02:	Top hat with ½ roll, ¼ rolls up an down (Turn-around manoeuvre) 	3

AP 03:	Two rolls in opposite direction (Centre manoeuvre) 	4

AP 04:	Stall turn with ¼ rolls up and down (Turn-around manoeuvre) 	3

AP 05:	Horizontal eight with ¼-roll, full roll, and ¾-roll integrated (Centre manoeuvre) 	5

AP 06:	½ horizontal knife edge square loop with 4/8-pt-roll (Turn-around manoeuvre) 	3

AP 07:	Knife edge cuban eight with ½ rolls (Centre manoeuvre) 	5

AP 08:	Half circle with ¾ roll integrated (Turn-around manoeuvre) 	4

AP 09:	1 1/2 torque rolls (Centre manoeuvre) 	5

AP 10:	Half square loop with ½ roll (Turn-around manoeuvre) 	3

AP 11:	45° downline with 2 quarter rolls (Centre manoeuvre) 	3

SCHEDULE F3P-AF		K-Factor

AF 01:	Clover leaf (Centre manoeuvre) 	4

AF 02:	Half horizontal square with 2/4-pt-rolls and ½ rolls integrated in the ¼ circles of corners (Turn-around man.) 	4

AF 03: 	Crossbox horizontal eight with 1/2-roll, two full roll opposite direction, and ½ roll integrated 	6

AF 04:	Stall turn combination (Turn-around manoeuvre) 	4

AF 05:	Loop with two full integrated rolls in opposite direction (Centre manoeuvre) 	5

AF 06:	Half square loop with full roll and ½ roll in opposite direction (Turn-around manoeuvre) 	3

AF 07:	Reverse cuban eight from top wit 2/4 pt roll and full roll (Centre manoeuvre) 	5

AF 08:	3/4-pt-roll followed by ½ knife edge loop down (Turn-around manoeuvre) 	5

AF 09:	Horizontal square loop with 1/2 rolls in each line (Centre manoeuvre) 	4

AF 10:	Stick back tail slide combination (Turn-around manoeuvre) 	5

AF 11:	Z with ½ rolls in all horizontal lines and full roll 45° upline (Centre manoeuvre) 	4

AF 12:	Half square loop with 2/4-pt-roll (Turn-around manoeuvre) 	3

AF 13:	1 ½ inverted snap roll (Centre manoeuvre) 	4

ANNEX 5M

F3P RADIO CONTROLLED AEROBATCS MODEL AIRCRAFT DESCRIPTION OF MANOEUVRES

Preliminary Manoeuvres – Schedule  F3P-AP

F3P AP 01: 	Lying square eight with ½ rolls in all vertical (up and down) lines 
From upright pull into a vertical upline, perform a half roll, push into a horizontal line, push into a vertical downline, perform a half roll, push into a horizontal line, push into a vertical upline, perform a half roll, pull into a horizontal line, pull into a vertical downline, perform a half roll , pull into a horizontal line, exit upright.

F3P AP 02:	Top hat with ½ roll, ¼ rolls up an down 
From upright pull into a vertical upline, perform ¼ roll, pull into a horizontal line crossbox, perform a half roll, push into a vertical downline, perform ¼ roll, push into a horizontal line, exit inverted.

F3P AP 03: 	Two rolls in opposite direction 
From inverted, fly two rolls with opposite direction, exit inverted.

F3P AP 04: 	Stall Turn with ¼ rolls up and down 
From inverted push into a vertical upline, perform ¼ roll, perform a stall turn, perform ¼ roll on downline, exit inverted.

F3P AP 05: 	Horizontal eight with ¼-roll, full roll, and ¾-roll integrated 
From inverted perform 90 degrees of a rolling circle with one quarter of a roll to the outside, integrated, followed immediately by a 360 degree circle in opposite direction with a full roll to the outside, integrated, followed immediately by a 270 degree circle in opposite direction, with three quarters of a roll to the outside, integrated, to exit inverted.

Judging notes:	The circles must have constant radii, and be of the same diameter · The altitude must not vary during the entire manoeuvre · The roll rates must be constant.

F3P AP 06: 	½ horizontal knife edge square loop with 4/8-pt-roll 
From inverted perform ¼ roll into knife edge, perform a 90 degree curve, perform 4 of an 8-pt-roll, perform a 90 degree curve, exit knife edge

F3P AP 07: 	Knife edge cuban eight with ½ rolls 
From knife edge, turn into 5/8 knife edge loop to a 45 degree downline, perform a half roll, turn into 3/4 knife edge loop to a 45 degree downline, perform a half roll, perform a 1/8 knife edge loop into a horizontal line, exit knife edge.

F3P AP 08: 	Half circle with ¾ roll integrated 
From knife edge perform a half circle with integrated ¾ roll, exit upright

F3P AP 09:	1 1/2 torque rolls 
From upright, reduce flying speed until the longitudinal axis of the model aircraft is in a vertically hovering attitude. Perform one and a half torque roll in this position then accelerate the model aircraft to exit inverted.

Judging notes:	The altitude and distance from the security line must be constant during the entire manoeuvre.

F3P AP 10: 	Half square loop with ½ roll 
From inverted push into a vertical upline, perform a half roll, push into a horizontal line, exit inverted.

F3P AP 11:	45degree downline with 2 quarter rolls 
From inverted, pull into a 45 degree downline, perform 2 of a 4-pt- roll, pull into horizontal line, exit upright

Finals Manoeuvres – Schedule  F3P-AF

F3P AF 01: 	Clover leaf 
From upright pull into a vertical upline, perform a ¾ inside loop into horizontal line, perform a ¾ inside loop into vertical downline, pull into horizontal line, exit upright

F3P AF 02: 	Half horizontal Square with 2/4-pt-rolls and ½ rolls integrated in the ¼ circles of corners 
From upright perform ¼ circle with integrated ½ roll into horizontal crossbox direction, perform ½ roll and 2/4-pt-roll in opposite direction, perform ¼ circle with integrated ½ roll into horizontal line, exit upright.

F3P AF 03: 	Crossbox horizontal eight with 1/2-roll, two full roll opposite direction, and ½ roll integrated 
From upright perform 180 degrees of a rolling circle with a half roll to the inside integrated, followed immediately by a 180 degree circle in opposite direction with a full roll to the inside integrated, followed immediately by a 180 degree circle with a full roll to the outside integrated, followed immediately by a 180 degree circle in opposite direction with a half of a roll to the inside integrated, to exit upright.

Judging notes:	The circles must have constant radii, and be of the same diameter. The altitude must not vary during the entire manoeuvre · The roll rates must be constant.

F3P AF 04: 	Stall Turn Combination 
From upright pull into vertical line, perform a stall turn followed by ¼ roll down, fly crossbox inverted perform ¼ roll to knife edge, turn into vertical upline, perform a stall turn, exit inverted

F3P AF 05: 	Loop with two full integrated rolls in opposite direction 
Perform an ½ outside loop with a full roll completely integrated, followed immediately by ½ outside loop with a full roll in opposite direction completely integrated

F3P AF 06: 	Half square loop with full roll and ½ roll in opposite direction 
Push to a vertical upline, perform one roll and a ½ roll in opposite direction and complete a half square loop.

F3P AF 07: 	Reverse cuban eight from top wit 2/4 pt roll and full roll 
Pull to a 45 degree downline and perform 2 of a 4-pt-roll. Then push through a 3/4 outside loop to a 45 degree downline, perform a full roll, pull through a 5/8 inside loop to exit inverted.

F3P AF 08: 	3/4-pt-roll followed by ½ knife edge loop down 
From inverted, perform 3 of a 4-pt-roll, perform a ½ knife edge loop down, exit knife edge

F3P AF 09: 	Horizontal square loop with 1/2 rolls in each line 
From knife edge, perform a ¼ circle to horizontal crossbox line, perform ½ roll, perform a ¼ circle to horizontal line, perform ½ roll, perform a ¼ circle to horizontal crossbox line, perform ½ roll, perform a ¼ circle to horizontal line, perform ½ roll, exit knife edge.

F3P AF 10: 	Stick back tail slide combination 
Turn into vertical line, followed by a stick-back tail slide fly inverted crossbox with two rolls opposite direction, push to vertical upline. perform ¼ roll followed by a inverted stick-back tail slide, exit upright

F3P AF 11: 	Figure Z with ½ roll in all horizontal lines and full roll 45degree upline 
From upright perform ½ roll, perform a 3/8 outside loop into a 45 degree upline, perform a roll, perform a 3/8 outside loop into a horizontal line, perform ½ roll, exit inverted

F3P AF 12: 	Half square loop with 2/4-pt-roll 
From inverted, pull into a vertical downline, perform 2of a 4-pt-roll, push into a horizontal line, exit inverted.

F3P AF 13: 	1 ½ inverted snap roll 
From inverted perform 1 ½ Snap, exit upright

End of document